Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

Aim

To analyze and demonstrate data communication, encapsulation, decapsulation, and packet routing across the OSI and TCP/IP reference models using a network simulation tool.

Objective

Study different network communication models (OSI 7-layer and TCP/IP 4-layer).

Analyze the packet flow and encapsulation process in a simulated enterprise network topology.

Compare the advantages and limitations of various network communication models.

Demonstrate real-time network traffic using Address Resolution Protocol (ARP), TCP 3-Way Handshake, and HTTP GET request workflows.

Selected Network Simulation Tool & Scenario

Selected Simulation Tool

Cisco Packet Tracer v8.x

Overview

A simulated network environment allows network administrators and engineers to:

Build multi-subnet network topologies.

Inspect layer-by-layer Protocol Data Units (PDUs) in real time.

Observe Address Resolution Protocol (ARP) execution.

Trace TCP 3-Way Handshake connection setup.

Analyze HTTP client-server data exchange and decapsulation.

Topology Setup & Components

Client Host Module

PC-A: IP 192.168.1.10 /24, Default Gateway: 192.168.1.1, MAC: 0060.2F11.AAAA

Switching Module

Switch S1: Cisco Catalyst 2960 (Layer 2 Switching)

Routing Module

Router R1: Cisco 1941 ISR (LAN: 192.168.1.1/24, WAN: 10.0.0.1/24)

Server Module

Server-A: IP 10.0.0.50 /24, Default Gateway: 10.0.0.1, MAC: 00D0.BA22.BBBB (HTTP Port 80 Active)

Network Communication Models Analysis

| Network Model | Suitability | Advantages | Limitations |
| --- | --- | --- | --- |
| OSI Model (7 Layers) | Medium | Comprehensive theoretical framework; clearly separates service, interface, and protocol concepts. | Complex implementation; redundant session/presentation layers; not universally deployed in commercial internet. |
| TCP/IP Model (4 Layers) | Very High | Pragmatic, highly scalable; universal internet standard; lightweight header overhead; simple 4-layer structure. | Does not clearly distinguish between service, interface, and protocol concepts. |

Why TCP/IP is Best for Real-World Implementation?

Reasons

Universal commercial adoption across all modern operating systems and networking hardware.

Streamlined 4-layer architecture minimizes protocol processing latency.

Seamless integration with underlying physical link-layer technologies (Ethernet, Wi-Fi, 5G).

Robust routing capabilities natively supported by modern internet infrastructure.

Data Encapsulation & Transmission Process

Application Data (HTTP Request)
        ↓
Transport Layer Encapsulation (TCP Segment Header Added)
        ↓
Internet Layer Encapsulation (IP Packet Header Added)
        ↓
Network Access Layer Encapsulation (Ethernet Frame Header & Trailer Added)
        ↓
Physical Bit Transmission (Raw Binary Signals over Media)
        ↓
Decapsulation at Destination (Server-A Processes HTTP Request)

Simulation Execution & Analysis Phases

1. Topology Configuration & IP Addressing

Setup network topology in Cisco Packet Tracer with PC-A, Switch S1, Router R1, and Server-A.

Deliverables: Network Topology Diagram, IP Addressing Scheme.

2. Address Resolution Protocol (ARP) Discovery

PC-A initiates communication with Server-A (10.0.0.50). Recognizes destination is outside local subnet and broadcasts ARP Request for Gateway 192.168.1.1.

Deliverables: ARP Request/Reply PDU Captures.

3. Connection Setup: TCP 3-Way Handshake

PC-A establishes a reliable connection with Server-A on Port 80.

SYN: PC-A sends SYN Segment (Seq=0).

SYN-ACK: Server-A replies with SYN-ACK (Seq=0, Ack=1).

ACK: PC-A sends ACK Segment (Seq=1, Ack=1). Session established.

4. HTTP Web Data Request & Decapsulation

PC-A issues an HTTP GET /index.html request formatted at Layer 7.

PDU Inspection Breakdown:

| OSI / TCP-IP Layer | PDU Header Component & Value |
| --- | --- |
| Application Layer | HTTP Payload: GET /index.html HTTP/1.1 |
| Transport Layer | TCP Header: Src Port: 49152, Dst Port: 80, Seq: 1, Ack: 1 |
| Internet Layer | IPv4 Header: Src IP: 192.168.1.10, Dst IP: 10.0.0.50, Protocol: 6 (TCP) |
| Network Access Layer | Ethernet Frame: Src MAC: 0060.2F11.AAAA, Dst MAC: 0002.4A11.1111 (Gateway R1) |

5. Routing & Hop-by-Hop Frame Rewriting

Router R1 receives the frame, strips Layer 2 header, inspects Layer 3 IP header, and re-encapsulates the IP packet into a new Ethernet frame with Destination MAC = Server-A (00D0.BA22.BBBB).

6. Decapsulation & Response Delivery

Server-A receives the raw physical bits, decapsulates bottom-up through Network Access, Internet, and Transport layers, delivering the HTTP payload to the Web Server application.

Comparison of Communication Models

| Criteria | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Layer Count | 7 Layers | 4 Layers |
| Approach | Theoretical / Conceptual | Practical / Implementation |
| Session & Presentation Layers | Explicitly defined as separate layers | Merged into Application Layer |
| Protocol Dependency | Protocol Independent | Protocol Dependent (TCP/IP Suite) |
| Header Processing Overhead | Higher (due to extra layering) | Lower (optimized for speed) |
| Suitable for Practical Networks | No (Used for teaching/reference) | Best (Universal internet standard) |

Results

The simulated web transaction successfully demonstrated multi-layer encapsulation and decapsulation across subnet boundaries.

TCP/IP 4-layer model provided efficient data delivery with minimal processing overhead.

Intermediate routers processed packets up to Layer 3 (IP header), changing MAC addresses hop-by-hop while maintaining end-to-end IP addresses.

Therefore, the TCP/IP model is the most practical and efficient standard for modern network communication.

Conclusion

The case study demonstrates that while the OSI 7-layer model provides an essential theoretical foundation for understanding network modularity, the TCP/IP 4-layer model serves as the practical, real-world framework driving modern internet communications. Network simulation tools like Cisco Packet Tracer effectively visualize layer-by-layer PDU encapsulation, enabling deep operational understanding of network protocol interactions.